| Content | Type | 2.5 RS Plug | 2.5 RS Connector | JDM Plug | JDM Connector |
| --- | --- | --- | --- | --- | --- |
| Crank Pos Sensor | Signal (+) | B135 | 1 | B136 | 5* or 6 |
| Crank Pos Sensor | Signal (-) | B135 | 8 | B136 | 23 |
| Crank Pos Sensor | Shield | B135 | 10 | B136 | 30 |
| Cam Pos Sensor | Signal (+) | B135 | 2 | B136 | 6* or 7 |
| Cam Pos Sensor | Signal (-) | B135 | 9 | B136 | 23 |
| Cam Pos Sensor | Shield | B135 | 10 | B136 | 30 |
| MAF | Signal | B136 | 5 | B136 | 1 |
| MAF | PSU | B136 | 15 | -------- | -------- -------- |
| MAF | Shield | B136 | 25 | B136 | 30 |
| MAF | GND | B136 | 8 | B136 | 31 |
| Throttle Pos Sensor | Signal | B136 | 17 | B136 | 20 |
| Throttle Pos Sensor | PSU | B136 | 15 | B136 | 12 |
| Throttle Pos Sensor | GND | B136 | 16 | B136 | 24 |
| Front O2 Sensor | Signal (+) | B136 | 7 | B136 | 21 |
| Front O2 Sensor | Signal (-) | B136 | 20 | -------- | -------- -------- |
| Front O2 Sensor | Shield | B136 | 23 | B136 | 30 |
| Rear O2 Sensor | Signal | B136 | 18 | -------- | ---------------- |
| Rear O2 Sensor | Shield | B136 | 24 | -------- | ---------------- |
| Rear O2 Sensor | GND | B136 | 16 | -------- | ---------------- |
| Front O2 Sensor Heater | Signal 1 | B134 | 22 | -------- | ---------------- |
| Front O2 Sensor Heater | Signal 2 | B134 | 23 | -------- | -------- -------- |
| Rear O2 Sensor Heater | Signal | B134 | 21 | -------- | -------- -------- |
| Engine Cool temp sensor | Signal | B136 | 14 | B136 | 28 |
| Engine Cool temp sensor | GND | B136 | 16 | B136 | 24 |
| Vehicle Speed Signal |  | B135 | 24 | B135 | 26 |
| Starter Switch |  | B135 | 28 | B135 | 2 |
| A/C Switch |  | B135 | 27 | B135 | 11 |
| Ignition Switch |  | B135 | 7 | B136 | 25 |
| Neutral Pos switch MT/AT |  | B135 | 26 | B135 | 29 |
| Test Mod Connector |  | B135 | 14 | B135 | 22 |
| Knock Sensor | Signal | B136 | 4 | B136 | 26 |
| Knock Sensor | Shield | B136 | 25 | B136 | 29 |
| AT/MT Identification |  | B135 | 25 | B135 | 4 |
| Back Up PSU |  | B136 | 9 | B135 | 17 |
| Control Unit PSU |  | B136 | 1 | B136 | 9 |
| Control Unit PSU |  | B136 | 2 | B136 | 8 |
| Sensor PSU |  | B136 | 15 | -------- | -------- -------- |
| Line end Check 1 |  | B135 | 20 | -------- | -------- -------- |
| Ignition Control | #1, #2 | B134 | 25 | B134 | 7 |
| Ignition Control | #3, #4 | B134 | 26 | B134 | 16 |
| Fuel Injector | #1 | B134 | 4 | B134 | 31 |
| Fuel Injector | #2 | B134 | 13 | B134 | 32 |
| Fuel Injector | #3 | B134 | 14 | B134 | 25 |
| Fuel Injector | #4 | B134 | 15 | B134 | 18 |
| Idle Air Control Valve | Signal 1 | B134 | 5 | B134 | 14 |
| Idle Air Control Valve | Signal 2 | B134 | 6 | B134 | 5 |
| Idle Air Control Valve | Signal 3 | B134 | 19 | B134 | 15 |
| Idle Air Control Valve | Signal 4 | B134 | 20 | B134 | 6 |
| Idle Air Control Valve | PSU | B136 | 2 | -------- | -------- -------- |
| Fuel Pump Relay Control |  | B134 | 16 | B134 | 1* or 21 |
| A/C Relay Control |  | B134 | 17 | B134 | 22 |
| Radiator fan relay 1 control |  | B134 | 3 | B134 | 4 |
| Radiator fan relay 2 control |  | B134 | 2 | B134 | 13 |
| Self-shutoff control |  | B135 | 19 | B136 | 32 |
| Malfunction indicator lamp |  | B134 | 11 | B134 | 28 |
| Engine speed output |  | B134 | 30 | B135 | 14 |
| Torque Control 1 Signal |  | B135 | 16 | B135 | 3 |
| Torque Control 2 Signal |  | B135 | 17 | B135 | 12 |
| Torque Control cut Signal |  | B134 | 31 | B135 | 15 |
| Mass air flow signal for AT |  | B136 | 11 | -------- | -------- -------- |
| Purge control solenoid valve |  | B134 | 2 | B135 | 7 |
| Power steering switch |  | B135 | 13 | B135 | 1 |
| Atmos press sensor | Signal | B136 | 29 | -------- | -------- -------- |
| Atmos press sensor | PSU | B136 | 15 | -------- | -------- -------- |
| Atmos press sensor | GND | B136 | 16 | -------- | -------- -------- |
| Press sources switch |  | B134 | 9 | B134 | 26 |
| Fuel temp sensor |  | B136 | 26 | -------- | -------- -------- |
| Fuel level sensor |  | B136 | 27 | -------- | -------- -------- |
| Fuel tank press sens | Signal | B136 | 12 | -------- | -------- -------- |
| Fuel tank press sens | PSU | B136 | 15 | -------- | -------- -------- |
| Fuel tank press sens | GND | B136 | 16 | -------- | -------- -------- |
| Fuel tank press control valve |  | B134 | 1 | -------- | -------- -------- |
| Drain Valve |  | B134 | 10 | -------- | -------- -------- |
| AT diagnosis |  | B135 | 4 | -------- | -------- -------- |
| Line end Check 2 |  | B135 | 21 | -------- | -------- -------- |
| GND | Sensors | B136 | 16 | B136 | 24 |
| GND | injectors | B134 | 7 | B134 | 9 |
| GND | ign sys | B134 | 27 | B134 | 17 |
| GND | PSU | B134 | 8 | B134 | 8 |
| GND | control sys | B136 | 21 | B136 | 18 |
| GND | control sys | B136 | 22 | B136 | 1* or 21 |
| GND O2 sensor heater 1 |  | B134 | 35 | -------- | -------- -------- |
| GND O2 sensor heater 2 |  | B134 | 34 | -------- | -------- -------- |
| Read Memory connector |  |  |  | B135 | 13 |
| Wastegate control |  |  |  | B134 | 19 |
| AT Load signal |  |  |  | B136 | 2 |
| Pressure Sensor |  |  |  | B136 | 7 |

RED = USDM only, GREEN = JDM only, *With immobilizer

Green connector (-): Backup psu, line end check 1

Geen connector (+): malfunction indicator lamp (?13.7ohms), ign switch,

Black 136 19 20 throttle pos sensor veh speed signal

Grey top left: Neutral Pos switch MT/AT  top right: AT/MT Identification NOTE: Bundled on both sides. DONE

White = o2 sens